Summary for week 7

2025/8/1

- This week. I learn about the XAS. X-ray absorption spectroscopy (XAS) is an established experimental technique for studying the electronic and local geometric structures of materials. As a short-range order structural probe, it can be applied to all states of matter: crystalline or amorphous solids, liquids and gases. The method is element selective and highly sensitive, with little compromise required to integrate complex sample environment set-ups. These characteristics make the technique suitable for applications in a range of scientific disciplines, from chemistry and catalysis to environmental science, materials science, physics, biology, medicine and cultural heritage. An XAS spectrum is obtained by measuring the modulation of the sample absorption coefficient as a function of the incident X-ray beam energy. Data are usually collected in transmission detection mode, although fluorescence and electron yield detection modes are often used. The XAS spectrum is divided into two regimes: X-ray absorption near-edge structure and extended X-ray absorption fine structure. In this Primer, an overview of XAS fundamentals is given, together with a description of the experimental set-ups, sample requirements, data analysis and possible applications.

- When we tried to scan Au surface in order to the feedthrough didn’t work. Unfortunately, while the outer parts of it were removed, we still couldn’t find out what was going wrong. Subsequently, the only solution to the problem is to vent every chamber and remove the whole feedthrough. So, I witnessed the whole process of venting chambers. Before venting with N2, they switch off all the pumps, gauges. Then, turn they remove wires, cables, control units and magnetics from the instrument, at the same time, valve between A, P chamber was opened as well. After that, similar to the process of venting load lock, they first casted a plastic bag onto it with a tube linked to N2 jar inside. Then opened the valve of N2 and loosened nuts under N2 atmosphere. Once the flange on load lock was completely loosened, we pulled up the upper part of flange, ensuring load lock is vented. After that, the valve liking the load lock and P chamber was slowly open. To judge whether chambers are vented completely, we can touch the bellow connecting two sides of the instrument, if it feels significantly softer than usual, we can assume that the pressure in chambers is the same as the air. Additionally, the reason why we use N2 to vent chambers is that N2 is easier for pumps to pump out compared to air.

Besides venting chambers, I also get to know about the mechanism as well as structure of feedthrough. It is divided into two parts, the part in the chamber is driven by a segment of bellow, which is curved in one end. The bellow will convert a force to the inner part when turning. By turning the outside part which connects to the bellow, we can operate objects in UHV from outside.

- As we vented all of the instruments. I experienced some parts of passing samples. The job requires patience and carefulness. Specially, when passed a gold foil from P chamber to load lock, two passing poles were hard to operate and aim in air pressure. So that I needed to adjust them carefully and for many times.